- What is the procedure for a toxic gas alarm?

  - Ignore alarm and keep working

  - Look for staff

- Stop working immediately and exit Cleanroom and wait for notification from staff

- Call 911

- Is it ok to work with hazardous chemicals without gloves or eye protection?

  - Yes

  - No

  - Sometimes

  - None of the above

- When are safety goggles required?

  - When working with any chemical

  - When operating equipment with UV or high intensity light sources

  - When spin coating wafers

  - All of the above

- Should you work in the Lab with open toe shoes?

  - Yes

  - No

  - Only in the summer time

- Is it ok to operate processing equipment without training?

  - Yes

  - No

  - Only if you already know how

    - How should you dispose of solvents?

      - Pour into solvent bench drain sumps

      - Pour down the drain

      - Pour into squirt bottles

      - None of the above

        - Is it ok to work in the Lab alone?

- Yes, If you are not using any chemicals.

- No

  - When pouring chemicals you must wear:

A.  Chemical resistant gloves

B.  Face Shield

C.  Apron

D.  All of the above

  - When diluting chemicals always:

A.  Add water to acid

B.  Add acid to water

C.  Solvents to acid

D.  None of the above

  - Is it ok to throw chemical bottles in the trash?

- Yes

- No

11.  Where should you discard Cleanroom wipes that have been exposed to chemicals?

- In the trash

- In appropriate ziploc waste collection bag

- In your pocket

12.  Is it ok to train your colleagues on equipment?

- Yes

- No, schedule for training with staff

- Sometimes

13.  If a gas cabinet alarms:

A. Call 911

B. Call the dean

C. Call staff, do not approach cabinet

14.  If contact is made with HF what should you NOT do?

A.  Contact Physician

B.  Wash with water

C.  Rub area vigorously

D.  Use special Calcium Gluconate gel

E.  Both B and D

15.  If you spill a bottle of chemical on the floor you should:

A.  Evacuate the area and immediately notify staff

B.  Remove clothing and immediately shower if exposed to chemical then cover with blanket

C.  If Eyes exposed to chemical, rinse in eyewash station

D.  All of the above